EXP2-4 Experimental Plan M1

Group members: Wang Yuhan, Zhang Yuchen, Zheng Junhao

Che Xuanming, Han Yujia, Wang Yixuan

Ⅰ. Introduction

This experiment aims to investigate and study the bearings that have problems in the Pacific Tower project and analyze the potential factors that cause the bearing performance to fail to meet the requirements. Based on the background information and the conversation, it has been confirmed that the influencing factors are mold temperature, placement time, accelerator replacement, and crosslinking agent ratio during the manufacturing process, which impact the quality of bearings. This plan involves designing a reasonable experimental plan to clarify the correlation between these factors and bearing quality problems and providing a basis for subsequent in-depth research and solution.

Ⅱ. Experiment design

1. Test factors

The experiment considers four key factors affecting bearing performance, each tested at two different settings as shown in Table 1. Since these settings are equally important from an experimental standpoint, testing all 24=16 possible combinations are necessary to fully understand how these parameters interact and determine the optimal configuration.

Table 1. Investigation factors and their Settings

| Factor | Notation | Detail |
| --- | --- | --- |
| mold temperature | T1 | 155℃ |
|  | T2 | 170℃ |
| placement time | t1 | 20min |
|  | t2 | 30min |
| accelerator replacement | A1 | From a new manufacturer |
|  | A2 | From an old manufacturer |
| crosslinking agent ratio | C1 | Mass ratio: 1:50 |
|  | C2 | Volume ratio: 1:50 |

2. Sample performance test

After making the samples according to the above steps, it is essential to test their properties to determine whether these factors impact them. Considering that there are two dangerous sources, namely, shear waves and primary waves, when an earthquake occurs, we decided to measure these two properties of the samples: vertical stiffness and horizontal shear strength. Vertical stiffness determines the load-bearing capacity and the amount of structural settlement, while horizontal shear strength is decisive in preventing buckling, slippage, and brittle fracture of materials.

Table 2 Experiments for testing properties [1] [2] [3]

| Properties | Test | Equipment | Methods |
| --- | --- | --- | --- |
| Vertical stiffness | ASTM D575 | Compression Testing Machine |  |
| Horizontal shear strength | ISO 1827 | Universal Testing Machine (UTM) + Shear Box Fixture |  |

3. Experimental Design: Experimental Conditions and Sample Quantity

To investigate the effects of four factors - temperature, time, accelerator, and amount of sulfur cross-linker - on the final results, including their pairwise interactions, we first define a "significant effect" as a scenario where altering a factor's value or type produces non-negligible differences in test outcomes. Due to the purpose of the experiment and constraints in manpower, material resources, and time, there is neither the necessity nor the means to conduct an in-depth exploration of the specific reasons for the influence of various factors. Only uniform effects (where altering a factor consistently produces non-negligible differences) are relevant for guiding subsequent improvements. Thus, we assume uniformity of effects: any comparison between two samples differing in a single factor will yield consistent conclusions about its influence. Under this assumption, any pairwise comparison suffices to determine whether a factor (or a pair of factors) has an effect, without requiring analysis of interaction types (e.g., synergistic enhancement or inhibitory suppression).

The required comparisons total C41+C42=10 combinations. Given that n samples can generate Cn-12=nn-12 pairwise comparisons, solving nn-12<10 yields n=5. This demonstrates that only 5 distinct experimental samples are needed to determine all single-factor and pairwise interaction effects under these assumptions.

Table 3. Design of processing conditions for experimental samples

| Sample number | Temperature (℃) | Time (s) | Accelerator | Amount of sulfur cross-linker |
| --- | --- | --- | --- | --- |
| 1 | 155 | 20 | New | Mass ratio 1:50 |
| 2 | 170 | 20 | New | Mass ratio 1:50 |
| 3 | 155 | 30 | New | Mass ratio 1:50 |
| 4 | 155 | 20 | Old | Mass ratio 1:50 |
| 5 | 155 | 20 | New | Volume ratio 1:50 |

Table 3 provides a feasible experimental design. Comparing samples 1 and 2 can determine the effect of temperature on the results; comparing samples 1 and 3 can determine the effect of time on the results; comparing samples 1 and 4 can assess the impact of accelerator on the results; comparing samples 1 and 5 can evaluate the effect of the amount of sulfur cross-linker on the results. Simultaneously, comparing samples 2 and 3 can assess the combined effect of temperature and time on the results; comparing samples 3 and 4 can determine the combined effect of time and accelerator on the results. Similarly, all two-factor combined impacts on the results can be determined through analogous comparisons. Based on the Blamestorming conversation, we identified potential incorrect values for temperature, time, accelerator, and amount of sulfur cross-linker. These values were then used as modified conditions to prepare samples and conduct experiments. Additionally, each sample's two types of performances were tested three times with averaged measurements, resulting in 3 × 5 ×2= 30 measurements.

Ⅲ. Resource estimation

Sample preparation equipment: Mold heating equipment, Compound mill (used for mixing rubber, sulfur crosslinking agent, accelerator)

Performance testing equipment: Compression Testing Machine, Universal Testing Machine (UTM) + Shear Box Fixture

Samples: 5

Material: Rubber substrate, Sulfur crosslinking agent, Accelerator, Mold lubricant

Manpower: Technical personnel: 2 (responsible for equipment operation and data recording). Analyzing data: 1 person. All operators need to be trained.

Time: Sample preparation: 3 hours.

Performance testing: for Vertical stiffness: 5×3×30min = 7.5h

for Horizontal shear strength: 5×3× 45min = 11.25h

Total: 5 days. 1 day for equipment calibration and maintenance time, 1day for sample preparation, 3 days for performance testing.

IV. Reference

[1]. Test Methods for Rubber Properties in Compression. [cited 2025 May 13]; Available from: https://img.antpedia.com/standard/files/pdfs_ora/20221211/astm/ASTM%20D575-91%20(2018).pdf

[2]. for O. ISO 1827:2016 [Internet]. ISO. 2016 [cited 2025 May 13]. Available from: https://www.iso.org/standard/70363.html?utm_source=chatgpt.com

[3]. QXU5017-B24 | MyQMUL [Internet]. Qmul.ac.uk. 2023. Available from: https://qmplus.qmul.ac.uk/pluginfile.php/4280177/mod_resource/content/9/EXP2-4%20Part1%20Introduction%2024B.pdf